SIMATS ENGINEERING

CO3-ASSESSMENT TOOL 1

Experiments

COURSE NAME: Computer Networks			COURSE CODE:	CSA07

COURSE OUTCOME COVERED

CO3: Analyze the performance of core network protocols such as TCP, UDP, HTTP, and DNS under varying conditions

Assessment Tool Weightage: 40%

Dr.V.Parthipan

Code of Conduct:

I _EKALAIVAN L(192524450) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student

EKALAIVAN.L

Experiments

QUESTIONS:4						Total Marks:40

- Capture packets during a TCP connection setup. Identify the three packets involved in the handshake (SYN, SYN-ACK, and ACK). Demonstrate the role of each flag and how they establish a reliable connection. (BL4)	                                                         (10)

- Capture a DNS query using UDP in Wireshark. Compare the UDP header size with TCP and note the absence of sequence numbers or acknowledgments. Discuss how this lightweight design impacts speed and reliability. (BL4)                   (10)

- Simulate packet loss (e.g., disconnect briefly) and capture traffic in Wireshark. Observe how TCP retransmits lost packets while UDP does not. Explain why this difference makes TCP reliable but slower compared to UDP. (BL4)             (10)

- Use Wireshark to capture DNS traffic while resolving a domain name. Identify the query type (A, AAAA, MX) and the response received. Measure the time taken for resolution and explain why DNS typically uses UDP.(BL4)                          (10)

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |

### Experiment 1: TCP three-way handshake

### Aim

Capture packets during a TCP connection setup and identify the SYN, SYN-ACK, and ACK packets. Explain how these flags establish a reliable connection.

### Why a handshake is required

TCP is a connection-oriented protocol. Before any application data (such as an HTTP request) is transmitted, both devices must agree on:

- the initial sequence numbers,

- the communication parameters,

- and that both sides are ready to send and receive data.

This agreement is performed using the three-way handshake.

TCP connection establishment

Client

Server

1. SYN

Seq = x

2. SYN-ACK

Seq = y, Ack = x + 1

3. ACK

Seq = x + 1, Ack = y + 1

Connection established

### Packet 1: SYN (Synchronize)

The client sends a TCP packet with the SYN flag set.

Example:

- Source: Client

- Destination: Server

- Flags: SYN

- Sequence Number: 1000 (example)

Purpose:

- request a connection,

- advertise the client’s initial sequence number (ISN),

- begin synchronization.

The sequence number is not random; it is generated by the operating system to improve reliability and security.

### Wireshark observation

In Wireshark:

- Flags = 0x002 (SYN)

- Acknowledgment number = 0

- Window size is advertised.

The TCP header contains:

- source port,

- destination port,

- sequence number,

- window size,

- TCP options.

### Packet 2: SYN-ACK

The server responds with both SYN and ACK flags set.

Example:

- Flags: SYN, ACK

- Sequence Number: 5000

- Acknowledgment Number: 1001

The acknowledgment number equals:

Client Seq+1\text{Client Seq} + 1Client Seq+1

because the SYN consumes one sequence number.

Purpose:

- acknowledge the client’s SYN,

- provide the server’s own ISN.

### Why does SYN consume one sequence number?

TCP treats SYN as a control event that occupies one position in the sequence space. This ensures that both sides can detect duplicate connection requests.

### Packet 3: ACK

The client acknowledges the server’s SYN.

Example:

- Flags: ACK

- Sequence Number: 1001

- Acknowledgment Number: 5001

After this packet:

- both sides know each other’s sequence numbers,

- both sides know the connection is established,

- data transfer can begin.

### How reliability is established

TCP reliability depends on sequence numbers and acknowledgments.

Suppose data is transmitted:

| Packet | Sequence number |
| --- | --- |
| Segment 1 | 1001 |
| Segment 2 | 1501 |
| Segment 3 | 2001 |

The receiver sends ACKs indicating the next expected byte.

If segment 2 is lost:

- receiver acknowledges only 1501,

- sender retransmits the missing segment.

Thus TCP guarantees:

- ordered delivery,

- duplicate detection,

- retransmission,

- flow control.

### Important TCP flags

| Flag | Purpose |
| --- | --- |
| SYN | Start connection |
| ACK | Acknowledge received data |
| FIN | Gracefully close connection |
| RST | Immediately terminate connection |
| PSH | Push data to application |
| URG | Urgent data |

### Why TCP is reliable

Because every byte is tracked using:

- sequence numbers,

- acknowledgments,

- retransmission timers,

- congestion control algorithms.

### Experiment 2: DNS query using UDP

### Aim

Capture a DNS query and compare the UDP header with TCP.

### What is DNS?

DNS converts domain names into IP addresses.

Example:

www.google.com

becomes

142.250.183.36

The client sends a DNS query to a DNS server.

### Why DNS uses UDP

DNS queries are usually:

- small,

- independent,

- request-response based.

Creating a TCP connection for every DNS query would require:

- SYN,

- SYN-ACK,

- ACK,

- data,

- FIN sequence.

That would increase latency significantly.

UDP allows:

- immediate transmission,

- minimal overhead.

### UDP header

UDP has only 8 bytes.

Structure:

UDP header (8 bytes)

Source port

Destination port

Length

Checksum

No:

- sequence number,

- acknowledgment number,

- window size,

- connection state.

### TCP header

Minimum 20 bytes.

Contains:

- sequence number,

- acknowledgment number,

- flags,

- window size,

- urgent pointer,

- options.

TCP therefore carries much more control information.

### Wireshark observation

For a DNS query:

- UDP source port: random ephemeral port

- Destination port: 53

The DNS packet contains:

- Transaction ID

- Flags

- Questions

- Answer count

Example:

Query:

www.google.com

Type:

A

Class:

IN

### Absence of sequence numbers

UDP simply sends the datagram.

If it is lost:

- no retransmission by UDP,

- no acknowledgment,

- no ordering.

Reliability is left to the application.

DNS resolvers usually handle this by:

- waiting briefly,

- retrying the query,

- contacting another DNS server.

### Speed advantage

UDP avoids:

- handshake,

- acknowledgments,

- connection maintenance.

Therefore DNS lookups are typically completed in a few milliseconds.

### Experiment 3: Packet loss and retransmission

### Aim

Observe how TCP retransmits lost packets while UDP does not.

### Packet loss

Packets may be lost because of:

- congestion,

- wireless interference,

- router queue overflow,

- physical link errors.

### TCP retransmission process

Suppose the sender transmits:

- Packet 1

- Packet 2

- Packet 3

Packet 2 is lost.

Receiver obtains:

- Packet 1

- Packet 3

The receiver cannot acknowledge Packet 3 because TCP requires ordered delivery.

It repeatedly sends:

ACK = next expected byte

This creates duplicate ACKs.

### Fast retransmission

When the sender receives three duplicate ACKs, it assumes packet loss and retransmits immediately.

Wireshark displays:

- TCP Dup ACK

- TCP Fast Retransmission

- TCP Retransmission

### Timeout retransmission

If duplicate ACKs are insufficient:

- sender waits for a retransmission timeout (RTO),

- retransmits the packet.

### Why retransmission causes delay

Retransmission increases:

- bandwidth usage,

- latency,

- processing overhead.

TCP must also reduce its sending rate.

### Congestion control

TCP interprets loss as a sign of network congestion.

It reduces the congestion window.

Common algorithms:

- Slow Start

- Congestion Avoidance

- Fast Recovery

Thus TCP becomes slower but protects the network from collapse.

### UDP behavior

UDP sends:

Packet 1

Packet 2

Packet 3

If Packet 2 is lost:

- receiver never receives it,

- sender is unaware,

- no retransmission occurs.

Applications such as:

- video streaming,

- voice calls,

- online gaming

often prefer occasional packet loss over retransmission delay.

### Reliability vs speed

| Feature | TCP | UDP |
| --- | --- | --- |
| Retransmission | Yes | No |
| Ordering | Yes | No |
| Reliability | High | Low |
| Latency | Higher | Lower |
| Overhead | High | Low |

### Experiment 4: DNS traffic analysis

### Aim

Capture DNS traffic, identify the query type, response, and resolution time.

### DNS query types

### A record

Returns an IPv4 address.

Example:

example.com

→

93.184.216.34

### AAAA record

Returns an IPv6 address.

Example:

2606:2800:220:1:248:1893:25c8:1946

### MX record

Returns the mail server responsible for the domain.

Example:

mail.example.com

### DNS resolution process

When you open a website:

- Browser checks local cache.

- OS checks DNS cache.

- Resolver sends a DNS query.

- DNS server replies.

- Browser connects to the IP address

### Wireshark analysis

Filter:

dns

You will see:

Query:

- Standard query

- Name

- Type

Response:

- Standard query response

- IP address

- TTL (Time to Live)

### Measuring resolution time

Wireshark provides timestamps.

Example:

Query:

10.100000

Response:

10.105500

Resolution time:

10.105500−10.100000=5.5 ms10.105500 - 10.100000 = 5.5 \text{ ms}10.105500−10.100000=5.5 ms

A lower time indicates:

- nearby DNS server,

- cached result,

- low network congestion.

### Why DNS usually uses UDP

UDP is ideal because:

- DNS packets are small,

- only one request and one response are needed,

- no connection establishment,

- lower CPU and bandwidth overhead.

However, DNS uses TCP when:

- responses exceed UDP size limits,

- zone transfers occur,

- DNSSEC data becomes large,

- UDP communication fails.

### High-scoring conclusion

For a Level 4 (Exemplary) answer, conclude with protocol behavior rather than only observations.

A strong conclusion is:

TCP establishes a reliable connection through the three-way handshake using sequence numbers and acknowledgments. Its retransmission and congestion-control mechanisms ensure reliable, ordered delivery, but increase latency and overhead. UDP eliminates connection establishment and acknowledgment mechanisms, resulting in an 8-byte header and significantly lower delay. DNS primarily uses UDP because domain-name resolution is typically a lightweight request-response operation, while TCP is used only when reliability or larger message sizes are required. Wireshark captures clearly demonstrate the trade-off between TCP’s reliability and UDP’s speed..